Imagecheck.py

import numpy as np

import cv2

import warnings

import tflite_runtime.interpreter as tflite

import RPi.GPIO as GPIO

import lcd

import time

warnings.filterwarnings("ignore")

# ===============================

# GPIO Setup

# ===============================

BUZZER = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(BUZZER, GPIO.OUT)

# ===============================

# Load TFLite Model

# ===============================

model_path = "/home/pi/Desktop/Raspberry PiBased MultiModal AI Edge System/image_model.tflite"

interpreter = tflite.Interpreter(model_path=model_path)

interpreter.allocate_tensors()

input_details = interpreter.get_input_details()

output_details = interpreter.get_output_details()

IMG_SIZE = 224

THRESHOLD = 0.5

# ===============================

# Image Path

# ===============================

image_path = "/home/pi/Desktop/Raspberry PiBased MultiModal AI Edge System/deepfake detection/Fake"

print("\nChecking Image File...\n")

lcd.display("Checking Image","")

try:

# ===============================

# Read Image

# ===============================

img = cv2.imread(image_path)

if img is None:

raise Exception("Image not found")

img = cv2.resize(img, (IMG_SIZE, IMG_SIZE))

img = img / 255.0

# Expand dimensions

img = np.expand_dims(img, axis=0)

# Convert datatype

img = img.astype(np.float32)

# ===============================

# Prediction

# ===============================

interpreter.set_tensor(input_details[0]['index'], img)

interpreter.invoke()

pred = interpreter.get_tensor(output_details[0]['index'])[0][0]

if pred >= THRESHOLD:

label = "REAL IMAGE"

confidence = pred * 100

lcd.display("Image Result","REAL")

GPIO.output(BUZZER, GPIO.LOW)

else:

label = "FAKE IMAGE"

confidence = (1 - pred) * 100

lcd.display("Image Result","FAKE")

GPIO.output(BUZZER, GPIO.HIGH)

print("FAKE IMAGE DETECTED")

time.sleep(3)

GPIO.output(BUZZER, GPIO.LOW)

print("--------------------------------")

print("Image File:", image_path)

print("Raw Score:", round(float(pred),3))

print("Prediction:", label)

print("Confidence:", round(float(confidence),2), "%")

except Exception as e:

lcd.display("Image Error","Check File")

GPIO.output(BUZZER, GPIO.LOW)

print("Error processing image:", e)

print("\nDetection Completed\n")

GPIO.cleanup()

Videocheck.py

import cv2

import numpy as np

import tflite_runtime.interpreter as tflite

import RPi.GPIO as GPIO

import lcd

import time

# ==========================

# GPIO SETUP

# ==========================

GPIO.setwarnings(False)

BUZZER = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(BUZZER, GPIO.OUT)

GPIO.output(BUZZER, GPIO.LOW)

# ==========================

# LOAD MODEL

# ==========================

model_path = "/home/pi/Desktop/Raspberry PiBased MultiModal AI Edge System/video_model.tflite"

interpreter = tflite.Interpreter(model_path=model_path)

interpreter.allocate_tensors()

input_details = interpreter.get_input_details()

output_details = interpreter.get_output_details()

IMG_SIZE = (224,224)

class_names = ["Fake","Real"]

# ==========================

# VIDEO PATH

# ==========================

video_path = "/home/pi/Desktop/Raspberry PiBased MultiModal AI Edge System/video dataset/Real/Real (1).mp4"

cap = cv2.VideoCapture(video_path)

fake_count = 0

real_count = 0

frame_id = 0

lcd.display("Video System","Processing")

print("\nVideo Detection Started\n")

# ==========================

# READ VIDEO

# ==========================

while True:

ret, frame = cap.read()

if not ret:

break

if frame_id % 5 == 0:

img = cv2.resize(frame, IMG_SIZE)

img = img / 255.0

img = np.expand_dims(img, axis=0).astype(np.float32)

interpreter.set_tensor(input_details[0]['index'], img)

interpreter.invoke()

prediction = interpreter.get_tensor(output_details[0]['index'])[0]

pred_class = np.argmax(prediction)

label = class_names[pred_class]

if label == "Fake":

fake_count += 1

color = (0,0,255)

else:

real_count += 1

color = (0,255,0)

cv2.putText(

frame,

label,

(30,40),

cv2.FONT_HERSHEY_SIMPLEX,

1,

color,

2

)

cv2.imshow("Video Detection", frame)

if cv2.waitKey(1) & 0xFF == 27:

break

frame_id += 1

cap.release()

cv2.destroyAllWindows()

# ==========================

# FINAL RESULT

# ==========================

print("\nFake Frames:", fake_count)

print("Real Frames:", real_count)

if fake_count > real_count:

print("Final Result: FAKE VIDEO")

lcd.display("Video Result","FAKE")

GPIO.output(BUZZER, GPIO.HIGH)

time.sleep(3)

GPIO.output(BUZZER, GPIO.LOW)

else:

print("Final Result: REAL VIDEO")

lcd.display("Video Result","REAL")

GPIO.output(BUZZER, GPIO.LOW)

GPIO.cleanup()

Audiocheck.py

import numpy as np

import librosa

import cv2

import warnings

import tflite_runtime.interpreter as tflite

import RPi.GPIO as GPIO

import lcd

import time

warnings.filterwarnings("ignore")

# ===============================

# GPIO Setup

# ===============================

BUZZER = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(BUZZER, GPIO.OUT)

# ===============================

# Load TFLite Model

# ===============================

model_path = "/home/pi/Desktop/Raspberry PiBased MultiModal AI Edge System/audio_model.tflite"

interpreter = tflite.Interpreter(model_path=model_path)

interpreter.allocate_tensors()

input_details = interpreter.get_input_details()

output_details = interpreter.get_output_details()

IMG_SIZE = 224

THRESHOLD = 0.87

# ===============================

# Audio Path

# ===============================

audio_path = "/home/pi/Desktop/Raspberry PiBased MultiModal AI Edge System/filtered_audio/fake/Fake (1).wav"

print("\nChecking Audio File...\n")

lcd.display("Checking Audio","")

try:

# ===============================

# Load audio (same as PC)

# ===============================

signal, sr = librosa.load(audio_path, sr=22050)

# ===============================

# Mel Spectrogram

# ===============================

mel = librosa.feature.melspectrogram(

y=signal,

sr=sr,

n_mels=128

)

mel_db = librosa.power_to_db(mel, ref=np.max)

# Normalize

mel_db = (mel_db - mel_db.min()) / (mel_db.max() - mel_db.min())

# Resize

mel_img = cv2.resize(mel_db, (IMG_SIZE, IMG_SIZE))

# Convert to 3 channels

mel_img = np.stack((mel_img,) * 3, axis=-1)

# Expand dimensions

mel_img = np.expand_dims(mel_img, axis=0)

# VERY IMPORTANT

mel_img = mel_img.astype(np.float32)

# ===============================

# Prediction

# ===============================

interpreter.set_tensor(input_details[0]['index'], mel_img)

interpreter.invoke()

pred = interpreter.get_tensor(output_details[0]['index'])[0][0]

if pred >= THRESHOLD:

label = "FAKE AUDIO"

confidence = pred * 100

lcd.display("Audio Result","FAKE")

GPIO.output(BUZZER, GPIO.HIGH)

print("FAKE AUDIO DETECTED")

time.sleep(3)

GPIO.output(BUZZER, GPIO.LOW)

else:

label = "REAL AUDIO"

confidence = (1 - pred) * 100

lcd.display("Audio Result","REAL")

GPIO.output(BUZZER, GPIO.LOW)

print("--------------------------------")

print("Audio File:", audio_path)

print("Raw Score:", round(float(pred),3))

print("Prediction:", label)

print("Confidence:", round(float(confidence),2), "%")

except Exception as e:

lcd.display("Audio Error","Check File")

GPIO.output(BUZZER, GPIO.LOW)

print("Error processing audio:", e)

print("\nDetection Completed\n")

GPIO.cleanup()

Lcd.py

import smbus

import time

I2C_ADDR = 0x27  # Your detected I2C address

LCD_WIDTH = 16

LCD_CHR = 1

LCD_CMD = 0

LCD_LINE_1 = 0x80

LCD_LINE_2 = 0xC0

E_PULSE = 0.0005

E_DELAY = 0.0005

bus = smbus.SMBus(1)

LCD_BACKLIGHT = 0x08

ENABLE = 0b00000100

def lcd_init():

lcd_byte(0x33, LCD_CMD)

lcd_byte(0x32, LCD_CMD)

lcd_byte(0x06, LCD_CMD)

lcd_byte(0x0C, LCD_CMD)

lcd_byte(0x28, LCD_CMD)

lcd_byte(0x01, LCD_CMD)

time.sleep(E_DELAY)

def lcd_byte(bits, mode):

high = mode | (bits & 0xF0) | LCD_BACKLIGHT

low = mode | ((bits << 4) & 0xF0) | LCD_BACKLIGHT

bus.write_byte(I2C_ADDR, high)

lcd_toggle_enable(high)

bus.write_byte(I2C_ADDR, low)

lcd_toggle_enable(low)

def lcd_toggle_enable(bits):

time.sleep(E_DELAY)

bus.write_byte(I2C_ADDR, bits | ENABLE)

time.sleep(E_PULSE)

bus.write_byte(I2C_ADDR, bits & ~ENABLE)

time.sleep(E_DELAY)

def lcd_print(message, line):

message = message.ljust(LCD_WIDTH, " ")

lcd_byte(line, LCD_CMD)

for char in message:

lcd_byte(ord(char), LCD_CHR)

def display(line1, line2):

lcd_init()

lcd_print(line1, LCD_LINE_1)

lcd_print(line2, LCD_LINE_2)

time.sleep(2)

if __name__ == "__main__":

display("Liquid Crystal", "Code Check")

threshold.py

import os

import numpy as np

import librosa

import cv2

import warnings

import tflite_runtime.interpreter as tflite

warnings.filterwarnings("ignore")

# ===============================

# Load TFLite Model

# ===============================

model_path = "/home/pi/Desktop/Tk196742_ Raspberry PiBased MultiModal AI Edge System/audio_model.tflite"

interpreter = tflite.Interpreter(model_path=model_path)

interpreter.allocate_tensors()

input_details = interpreter.get_input_details()

output_details = interpreter.get_output_details()

IMG_SIZE = 224

# ===============================

# Dataset Paths

# ===============================

real_path = "/home/pi/Desktop/Tk196742_ Raspberry PiBased MultiModal AI Edge System/filtered_audio/real"

fake_path = "/home/pi/Desktop/Tk196742_ Raspberry PiBased MultiModal AI Edge System/filtered_audio/fake"

# ===============================

# Preprocessing Function

# ===============================

def preprocess(audio_file):

signal, sr = librosa.load(audio_file, sr=22050)

mel = librosa.feature.melspectrogram(

y=signal,

sr=sr,

n_mels=128

)

mel_db = librosa.power_to_db(mel, ref=np.max)

mel_db = (mel_db - mel_db.min()) / (mel_db.max() - mel_db.min())

mel_img = cv2.resize(mel_db, (IMG_SIZE, IMG_SIZE))

mel_img = np.stack((mel_img,) * 3, axis=-1)

mel_img = np.expand_dims(mel_img, axis=0).astype(np.float32)

return mel_img

# ===============================

# Collect Scores

# ===============================

real_scores = []

fake_scores = []

print("\nChecking REAL audios...\n")

for file in os.listdir(real_path):

if file.endswith(".wav"):

path = os.path.join(real_path, file)

img = preprocess(path)

interpreter.set_tensor(input_details[0]['index'], img)

interpreter.invoke()

pred = interpreter.get_tensor(output_details[0]['index'])[0][0]

real_scores.append(pred)

print(file, "Score:", round(float(pred),3))

print("\nChecking FAKE audios...\n")

for file in os.listdir(fake_path):

if file.endswith(".wav"):

path = os.path.join(fake_path, file)

img = preprocess(path)

interpreter.set_tensor(input_details[0]['index'], img)

interpreter.invoke()

pred = interpreter.get_tensor(output_details[0]['index'])[0][0]

fake_scores.append(pred)

print(file, "Score:", round(float(pred),3))

# ===============================

# Threshold Testing

# ===============================

print("\n\nTesting thresholds...\n")

best_acc = 0

best_threshold = 0

for t in np.arange(0.1, 0.9, 0.01):

correct = 0

total = 0

for s in real_scores:

if s < t:

correct += 1

total += 1

for s in fake_scores:

if s >= t:

correct += 1

total += 1

acc = correct / total

if acc > best_acc:

best_acc = acc

best_threshold = t

print("\n--------------------------------")

print("Best Threshold:", round(best_threshold,3))

print("Best Accuracy:", round(best_acc*100,2), "%")

##print("--------------------------------")